FACULTY OF ENGINEERING AND TECHNOLOGY

### A REPORT ABOUT HYDROLOGICAL CATCHMENT DELINEATION AND RUN-OFF MEASUREMENT PROJECT & A DEVELOPED PROFFESSIONAL GRAPHICAL USER INTERFACE (GUI).

### COURSE UNIT: COMPUTER PROGRAMMING

### LECTURER: Mr. MASERUKA BENEDICTO

PREPARED AND SUBMITTED BY GROUP 11

# ACKNOWLEDGEMENT

We thank God for enabling us take on the challenge despite the toughness. Gratitude goes to all the members of group 11 who have sacrificed to make it this far. The will power of each one of us to invest time and provide constructive feedback has been immensely valuable in this assignment. Finally, we would like to express gratitude to the lecturer Mr. MASERUKA BENEDICTO, you have granted us resourceful information and material that has aided us in the assignment.

# DEDICATION.

We dedicate this report to our friends and loved ones who have seen us through the process of doing the assignment, formulating and compiling the report. We also dedicate it to our lecturer Mr. Maseruka Benedicto, whose guidance and expertise have been invaluable, your mentorship and insightful feedback have shaped our understanding of the course unit.

# DECLARATION

We members of GROUP 11 hereby declare and confirm that the information in this report is our own information generated and compiled by the group members. It has never been submitted by anyone in any University or institution of higher learning. The report contents have been compiled with generous academic integrity following the parameters presented and organized by group 11 and any academic decision taken based on the report contents is however based on the group responsibility.

## GROUP ELEVEN MEMBERS.

| NAME | REGISTRATION NUMBER | PROGRAM OF STUDY | SIGNATURE |
| --- | --- | --- | --- |
| MUKYALA ROSE MIIRIAM | BU/UP/2024/1041 | WAR |  |
| OGWANG EMMANUEL | BU/UP/2024/3254 | WAR |  |
| NSAMBA SHAFFIC | BU/UP/2024/5346 | MEB |  |
| KAGODA ENOCK | BU/UP/2023/1097 | WAR |  |
| BUSUULWA HASSAN | BU/UP/2024/3732 | AMI |  |
| WAMBI KENNETH | BU/UP/2023/0950 | PTI |  |
| ATWIINE SANDRAH | BU/UG/2024/2632 | MEB |  |
| ABESIGA KEITH ADAMS | BU/UP/2024/3974 | AMI |  |
| WASSWA FRANCIS | BU/UG/2024/2669 | APE |  |
| NANDERA JULIET | BU/UP/2024/1047 | WAR |  |

DATE OF SUBMISSION: ………………………………

SUBMITTED TO: …………………………………

SIGNATURE: ……………………………………………..

# TABLE OF CONTENTS.

ACKNOWLEDGEMENT	2

DEDICATION.	3

DECLARATION	4

GROUP ELEVEN MEMBERS.	4

TABLE OF CONTENTS.	5

APPROVAL	9

LIST OF ACRONYMS AND ABBREVIATIONS.	10

ABSTRACT	11

CHAPTER ONE:	12

1.1 Background.	12

1.2 Historical Development	12

•	Initial Development	12

•	Commercial Launch	12

•	Expansion and Toolboxes	12

•	Modern Enhancements	12

CHAPTER 2: STUDY METHODOLOGY	13

2:1 Introduction.	13

2.2 DISCUSSION CHAPTER 1.	13

2.2.0 HYDROLOGICAL CATCHMENT DELINEATION	13

METHODS:	13

Digital/GIS Method (Most Accurate):	13

2.2.1. RUN-OFF MEASUREMENT	14

Methods:	14

2.2.3. Modeling and Estimation	14

2.2.4. PROJECT COMPONENTS	15

1.	Data Collection	15

Catchment Delineation:	15

Run-off Analysis:	15

Reporting and Application:	15

2.2.5 WRITING THE MATLAB CODE.	16

2.2.6 TESTING AND VALIDATING THE CODE.	16

2.2.6 VISUALIZATION AND INTERPRETATION OF THE RESULTS.	16

2.2.7 CODE GENERATION	16

2.2.8   STEPS INVOLVED.	17

Step.1 data set identification.	17

Step.2 Preprocess.	18

Step.3. DEM pour point.	20

Step.4.Delineate catchment	22

Step.5. Compute area.	23

2.2.9 CLASSES & PARENT CLASSCODES INVOLVED IN THE PROJECT.	24

CATCHMENTMODEL.	24

HYDROMODEL	25

RATIONALMODEL	26

RUN-SIMULATION FROM DATA SET.	26

SOIL CONSERVATION SERVICE CURVE NUNMER METHOD (SCS)	28

2.2.10 GRAPHICAL USER INTERFACE.	29

This s an application we designed to run specific interface for the hydrological datasets of the project.   It was also to run different input data coded in to MATLAB.WE designed it in order to simplify and quickly have targeted data access about the project.	29

CONCLUSION	33

REFERENCES	33

# APPROVAL

This is to approve that this report contains the genuine work done by GROUP 11 .

LECTURER

NAME………………………………………………………………………………..

DATE………………………………………………………………………………...

SIGNATURE……………………………………………………………………..….

# LIST OF ACRONYMS AND ABBREVIATIONS.

- GUI - Graphical user interface.

- MATLAB - Matrix Laboratory.

- DEM  -  Digital Elevation Model

- GIS – Geographical Information System

# ABSTRACT

## This report contains information pertaining to the different things we did to ensure that we finish the assignment successfully. We convened a meeting and strategized on how to do the assignment. We divided tasks to ease the collection of data, writing of codes and compilation of the report as well as coming up with the presentation.  We were required to develop the hydrological catchment delineation and run-off measurement project. And generating a graphical User Interface

Hydrological catchment delineation and run-off measurement projects involve defining the land area that contributes water to a specific outlet and quantifying the water flowing from it. This work is essential for water resource management, hydraulic structure design, and flood prediction.

# CHAPTER ONE:

## 1.1 Background.

MATLAB, which stands for matrix laboratory, is a high-performance programming language and environment designed primarily for technical computing. Its origins trace back to the late 1970s when Cleve Moler, a professor of computer science, developed it to provide his students with easy access to mathematical software libraries without requiring them to learn Fortran.

### 1.2 Historical Development

- Initial Development: The first version of MATLAB was created in Fortran in the late 1970s as a simple interactive matrix calculator. This early iteration included basic matrix operations and was built on top of two significant mathematical libraries: LINPACK and EISPACK, which were developed for numerical linear algebra and eigenvalue problems, respectively.

- Commercial Launch: MATLAB was officially launched as a commercial product in

1984 by Math Works, a company founded by Moller along with Jack Little and Steve Bangert. This marked the transition from a simple calculator to a comprehensive programming environment. The software was implemented in C, enhancing its capabilities with the addition of user-defined functions, toolboxes, and graphical interfaces.

- Expansion and Toolboxes: Over the years, MATLAB has expanded significantly. By the late 1980s, it had introduced several specialized toolboxes for various applications, including control systems and signal processing. The introduction of the Simulink environment further allowed users to model and simulate dynamic systems graphically.

- Modern Enhancements: Recent versions of MATLAB have introduced features like the Live Editor, which allows users to create interactive documents that combine code, output, and formatted text. This evolution reflects MATLAB's ongoing adaptation to meet the needs of its diverse user base across academia and industry.

# CHAPTER 2: STUDY METHODOLOGY

## 2:1 Introduction.

At the wake of the formation of the groups, each member joined in and straight away was given a task of making research about the assignment before our first meeting. The research was guided by the knowledge of the various resourceful materials at our exposure such as the notes and online tutorials on YouTube and websites like kaggle.com

## 2.2 DISCUSSION CHAPTER 1.

## This involves different research key terminologies, systems, design and methods of data collection carried out by group11 members about the hydrological catchment delineation and run-off measurement project

## 2.2.0 HYDROLOGICAL CATCHMENT DELINEATION

Catchment (watershed or drainage basin) delineation is the process of defining the boundary where all precipitation drains to a single, common outlet or pour point.

### METHODS:

- Manual Method:

Involves using topographic maps, identifying high points (ridges) and drawing boundary lines perpendicular to the contour lines, ensuring the boundary does not cross any streams except at the outlet.

### Digital/GIS Method (Most Accurate):

Uses a Digital Elevation Model (DEM) and specialized software (e.g., ArcGIS, QGIS, HEC-HMS, and WEAP). The general steps include:

  - Acquire and condition the DEM:

Fill sinks (correct imperfections in the elevation data).

#### Calculate Flow Direction:

Determine the path water will take from each cell (pixel) in the DEM.

#### Calculate Flow Accumulation:

Determine the number of upstream cells that drain into each cell; high accumulation areas usually indicate stream channels.

#### Define the Pour Point (Outlet):

Specify the exact location on the stream network for which the catchment is being delineated (e.g., a dam, a bridge, or a gauge station).

#### Run the Watershed Tool:

The software uses the flow direction and pour point data to automatically generate the catchment boundary as a raster, which can then be converted into a polygon for area calculation and further analysis.

## 2.2.1. RUN-OFF MEASUREMENT

Run-off measurement quantifies the volume and rate of surface water flow. This can be done in the field (direct measurement) or through modeling (estimation).

### Methods:

#### 2.2.2. Direct Field Measurements:

##### Area-Velocity Method:

Involves measuring the cross-sectional area of a river and the water velocity (using current meters or flow probes) at various points to calculate discharge (volume per time).

  - Weirs and Flumes:

Installing standardized hydraulic structures (weirs or flumes) in a channel, where flow rate can be determined by measuring the water level (stage).

  - Tracer Dilution Methods:

Injecting a known concentration of a chemical tracer (like salt) into a stream and measuring its dilution downstream to calculate flow rate, useful for turbulent or irregularly shaped channels where current meters are difficult to use.

  - Water Level Recorders:

Using instruments like ultrasonic sensors or float-and-counterweight systems in stilling wells to continuously record water levels, which are then correlated with discharge measurements to create a rating curve.

2.2.3. Modeling and Estimation:

#### Runoff Coefficient Method (Rational Method):

A simple empirical method that uses a coefficient (C) representing the land cover's permeability to estimate runoff volume from precipitation.

##### SCS Curve Number (SCS-CN) Method:

A widely used method by the Natural Resources Conservation Service to estimate the depth of direct surface runoff for a given rainfall event, considering soil type and land use.

##### Hydrological Models (e.g., HEC-HMS, SWAT):

# Computer models that simulate the entire hydrological cycle, using various data inputs (precipitation, land cover, soil type, DEM) to generate detailed runoff hydrographs and predict flood events.

# 2.2.4. PROJECT COMPONENTS

A typical project would involve:

- Data Collection:

Gathering topographic maps or a high-resolution DEM, land use/cover data, soil data, and long-term rainfall data from weather stations. Data was obtained from different websites like kaggle.com

## Catchment Delineation:

Using the methods above to define the study area boundaries and sub-basins.

## Run-off Analysis:

Measuring or modeling run-off to understand the quantity, quality, and timing of water flow, often resulting in a runoff hydrograph.

## Reporting and Application:

Using the results for planning drainage infrastructure, designing flood control measures, or assessing water availability.

#### 2.2.5 GRAPHICAL USER INTERFACE (GUI):

### This is an application developed to run different input data coded in to MATLAB.WE designed it in order to simplify and quickly have targeted data access about the project.

# 2.2.5 WRITING THE MATLAB CODE.

We wrote a MATLAB script that implements the chosen numerical methods.it involved defining the functions and DEs, specifying the initial conditions, implementing the numerical method, storing and plotting the results.

## 2.2.6 TESTING AND VALIDATING THE CODE.

We tested the MATLAB code with known solutions to validate the results by comparing the numerical solution with an exact solution.

## 2.2.6 VISUALIZATION AND INTERPRETATION OF THE RESULTS.

We plotted graphs using the plot function and added various components to them.

As well we added brief analysis of the behavior of the solutions, identifying patterns and drawing conclusions based on the visuals and results.

# 2.2.7 CODE GENERATION

The code was generated following the ideas of classes, subclasses, encapsulation, inheritance, and polymorphism depending on the different datasets obtained from Kaggle.com based on a land site DEM dataset of Rwanda and Sponge –Village Run-off dataset.

We needed to build a hydrological simulation system capable of:

- Loading rainfall–runoff datasets.

- Loading DEM (Digital Elevation Model) data.

- Processing the data visually and numerically.

- Designing a MATLAB GUI that integrates these functions

# 2.2.8   STEPS INVOLVED.

## Step.1 data set identification.

It represents the data requirements and problem analysis phase were we examined the structure, type and format hydrological data sets before building any code

- Rainfall dataset (Excel file)

- DEM dataset folder

% step1_inspect_data.m

clear; close all; clc;

% --- CSV preview ---

csvFile = "sponge_village_dataset.csv";

T = readtable(csvFile);

disp("CSV size:"); disp(size(T));

disp("Columns:"); disp(T.Properties.VariableNames);

disp("First 6 rows:"); disp(T(1:6,:));

% --- DEM preview ---

demFile = fullfile("DEM","dem_Rwanda_10m.tif");

if exist(demFile,'file')

try

[Z, R] = readgeoraster(demFile);

Z = double(Z);

fprintf('DEM size: %d x %d\n', size(Z,1), size(Z,2));

figure('Name','DEM preview'); imagesc(Z); axis image; colorbar; title('DEM (elevation)');

catch

warning('Could not read DEM with readgeoraster. Try geotiffread or check path.');

end

else

warning('DEM file not found: %s', demFile);

end

## Step.2 Preprocess.

It involves creating a clean DEM to correctly compute run-off, flow routing, watershed shape and drainage networks

% step2_preprocess_csv.m

clear; clc;

csvFile = "sponge_village_dataset.csv";

T = readtable(csvFile);

% Ensure Date is datetime

if ~isdatetime(T.Date)

try

T.Date = datetime(T.Date,'InputFormat','yyyy-MM-dd'); % try common format

catch

T.Date = datetime(T.Date); % generic

end

end

% Rename columns to convenient names (if needed)

% Example uses the existing names: Rainfall_mm, Observed_Runoff_m3s, Infiltration_Rate_mm_h, Impervious_Surface_pct

% Fill or remove NaNs

fprintf('NaN count per column:\n');

disp(sum(ismissing(T)));

% For modelling we will drop rows without Rainfall or Observed_Runoff

T = T(~(ismissing(T.Rainfall_mm) | ismissing(T.Observed_Runoff_m3s)), :);

fprintf('Rows after dropping missing rainfall/runoff: %d\n', height(T));

% Convert units if necessary (e.g., area later)

% Quick plots:

figure; plot(T.Date, T.Rainfall_mm); ylabel('Rainfall (mm)'); xlabel('Date'); title('Rainfall time series');

figure; plot(T.Date, T.Observed_Runoff_m3s); ylabel('Observed Runoff (m^3/s)'); xlabel('Date'); title('Observed runoff');

## Step.3. DEM pour point.

It inputs and defines the outlet (pour points) of the catchment (Rwanda DEM). It also indicates the exact location of the pour point and a hydrologically-correct DEM to the actual delineate water shed

clear; clc; close all;

demFile = fullfile("DEM","dem_Rwanda_10m.tif");

[Z, R] = readgeoraster(demFile); % Z = elevation matrix, R = referencing object

Z = double(Z);

Z(Z==R.CellExtentInWorldY*0) = NaN; % just in case of weird no-data (adjust logic as needed)

hf = figure('Name','DEM - click pour point (select outlet/pour point)');

imagesc(Z); axis image; colorbar; title('DEM - click pour point');

set(gca,'YDir','normal'); % keep coordinate orientation consistent

[xpix, ypix] = ginput(1); % user clicks once

% Convert pixel to world coordinates:

[xWorld, yWorld] = intrinsicToWorld(R, xpix, ypix);

fprintf('Clicked intrinsic coords: (%.1f, %.1f)\n', xpix, ypix);

fprintf('World coords (X, Y): (%.6f, %.6f)\n', xWorld, yWorld);

% Save pour point location

pourPoint = struct('intrinsicX', xpix, 'intrinsicY', ypix, 'worldX', xWorld, 'worldY', yWorld);

save('pourPoint.mat','pourPoint');

clear; clc; close all;

demFile = fullfile("DEM","dem_Rwanda_10m.tif");

[Z, R] = readgeoraster(demFile); % Z = elevation matrix, R = referencing object

Z = double(Z);

Z(Z==R.CellExtentInWorldY*0) = NaN; % just in case of weird no-data (adjust logic as needed)

hf = figure('Name','DEM - click pour point (select outlet/pour point)');

imagesc(Z); axis image; colorbar; title('DEM - click pour point');

set(gca,'YDir','normal'); % keep coordinate orientation consistent

[xpix, ypix] = ginput(1); % user clicks once

% Convert pixel to world coordinates:

[xWorld, yWorld] = intrinsicToWorld(R, xpix, ypix);

fprintf('Clicked intrinsic coords: (%.1f, %.1f)\n', xpix, ypix);

fprintf('World coords (X, Y): (%.6f, %.6f)\n', xWorld, yWorld);

% Save pour point location

pourPoint = struct('intrinsicX', xpix, 'intrinsicY', ypix, 'worldX', xWorld, 'worldY', yWorld);

save('pourPoint.mat','pourPoint');

Step.4.Delineate catchment.

It creates the boundary of the catchment based on the DEM and pour point. It ensures that hydrological models must run only on the catchment area, not the whole DEM.

% step4_delineate_catchment.m

clear; clc; close all;

demFile = fullfile("DEM","dem_Rwanda_10m.tif");

[Z, R] = readgeoraster(demFile);

Z = double(Z);

Z(isnan(Z)) = nanmean(Z(~isnan(Z))); % simple fill

% Smooth DEM a little to remove noise

Zs = imgaussfilt(Z, 2);

% Compute a catchment mask via watershed on inverted DEM with marker at pour point

load('pourPoint.mat','pourPoint');

intrX = pourPoint.intrinsicX; intrY = pourPoint.intrinsicY;

% Build a marker image:

marker = false(size(Zs));

ix = round(intrY); iy = round(intrX); % careful: intrinsicToWorld mapping may invert axes

% But our earlier code saved xpix,y pix to intrinsicX/intrinsicY; check mapping:

% Here we assume intrinsicX is column index and intrinsicY is row index (swap if needed)

if ix < 1 || ix > size(Zs,1) || iy < 1 || iy > size(Zs,2)

% if swapped do this:

ix = round(intrX); iy = round(intrY);

end

marker(ix, iy) = true;

% Create imposed minima on inverted DEM and run watershed

D = imcomplement(Zs);                % invert so basins become peaks

D2 = imimposemin(D, marker);

L = watershed(D2);

basinMask = (L == L(ix,iy));        % mask corresponding to the basin where the marker sits

% Plot results

figure; imagesc(Z); axis image; colorbar; hold on;

h = imshow(basinMask); set(h,'AlphaData',0.3);

title('Delineated basin overlay (transparent)');

## Step.5. Compute area.

It quantifies the characteristics needed by hydrological models for computing catchment physical properties..

% step5_compute_area_props.m

clear; clc;

demFile = fullfile("DEM","dem_Rwanda_10m.tif");

[Z, R] = readgeoraster(demFile);

load('pourPoint.mat','pourPoint');

% assume basinMask computed in previous script, or recompute here by calling step4

% (for brevity let's assume basinMask is in workspace; otherwise run step4 first)

% compute cell area in m^2:

cellArea = abs(R.CellExtentInWorldX * R.CellExtentInWorldY); % approximate

nCells = sum(basinMask(:));

area_m2 = nCells * cellArea;

area_ha = area_m2 / 10000;

fprintf('Catchment area: %.1f ha (%.0f m^2)\n', area_ha, area_m2);

% Derive mean slope etc (simple)

[fx, fy] = gradient(Z);

slope = atan(sqrt(fx.^2 + fy.^2)); % radians

meanSlope_deg = mean(slope(basinMask),'omitnan') * (180/pi);

fprintf('Mean slope (deg): %.2f\n', meanSlope_deg);

It computes the Rational and SCS models that we used for hydrological quantification as shown below

# 2.2.9 CLASSES & PARENT CLASSCODES INVOLVED IN THE PROJECT.

## CATCHMENTMODEL.

classdef Catchment

properties (SetAccess = private)

Area_m2

Area_ha

Mask % logical mask of the basin (matrix)

DEM % elevation matrix

Ref % referencing object

MeanSlope_deg

end

methods

function obj = Catchment(mask, DEM, Ref)

obj.Mask = mask;

obj.DEM = DEM;

obj.Ref = Ref;

cellArea = abs(Ref.CellExtentInWorldX * Ref.CellExtentInWorldY);

obj.Area_m2 = sum(mask(:)) * cellArea;

obj.Area_ha = obj.Area_m2 / 10000;

% compute mean slope

[fx, fy] = gradient(DEM .* double(mask));

s = atan(sqrt(fx.^2 + fy.^2)) * (180/pi);

obj.MeanSlope_deg = mean(s(mask),'omitnan');

end

function a = getAreaHa(obj)

a = obj.Area_ha;

end

function plot(obj)

figure; imagesc(obj.DEM); axis image; hold on;

h = imshow(obj.Mask); set(h,'AlphaData',0.35);

title(sprintf('Catchment (Area = %.1f ha)', obj.Area_ha));

end

end

end

## HYDROMODEL

This parent class controls the other subclasses.

classdef (Abstract) HydroModel

% Abstract base for hydrological models

properties (Access = protected)

catchment % handle to Catchment object

end

methods

function obj = HydroModel(catchment)

obj.catchment = catchment;

end

end

methods (Abstract)

runoff = calculateRunoff(obj, rainfall_mm)

plotResults(obj, t, runoff)

end

end

## RATIONALMODEL

It is a sub-class implementing the rational run-off formular.it inherits from the parent class, which is the hydro model.

It implements the rational runoff formula i.e. Q = CIA

classdef RationalModel < HydroModel

properties

C % runoff coefficient (0-1)

end

methods

function obj = RationalModel(catchment, C)

obj@HydroModel(catchment);

obj.C = C;

end

function Q = calculateRunoff(obj, intensity_mm_per_h, duration_h)

% Q = C * I * A  (I in mm/hr, A in hectares -> convert appropriately)

A_ha = obj.catchment.Area_ha;

% Q (m3/s) = C * I (mm/h) * A_ha * (10000 m2/ha) / 3600 (s/h) /1000 (mm->m)

Q_m3s = obj.C .* intensity_mm_per_h .* (A_ha * 10000) ./ 3600 ./ 1000;

Q = Q_m3s;

end

function plotResults(obj, t, Q)

figure;

plot(t, Q); ylabel('Q (m^3/s)'); xlabel('Time'); title(sprintf('Rational model (C=%.2f)', obj.C));

end

end

end

## RUN-SIMULATION FROM DATA SET.

It is a functional model (method file) that acts as a controller between the GUI and the hydrological model class. It uses composition by creating objects from rational model or SCS model and executing their methods

% run_simulation_from_dataset.m

clear; clc;

% --- Load CSV

T = readtable("sponge_village_dataset.csv");

T.Date = datetime(T.Date);

% For the example pick a storm event subset (e.g., top rainfall events)

[sortedRain, idx] = sort(T.Rainfall_mm,'descend');

nEvents = 10;

idx = idx(1:nEvents);

events = T(idx,:);

% --- Load DEM and basinMask (assume step4 produced basinMask and saved)

demFile = fullfile("DEM","dem_Rwanda_10m.tif");

[Z, R] = readgeoraster(demFile);

load('basinMask.mat','basinMask'); % if you saved it; otherwise run step4 and compute basinMask

C = Catchment(basinMask, Z, R);

fprintf('Catchment area = %.1f ha\n', C.getAreaHa());

% --- SCS modelling: allow user to supply CN or estimate it

% Here estimate CN from Impervious_Surface_pct (very rough):

est_CN = 30 + 0.7 * events.Impervious_Surface_pct; % simple linear estimate => adjust as needed

% Or pick a constant CN:

CN = mean(est_CN,'omitnan');

scs = SCSModel(C, CN);

runoff = scs.calculateRunoff(events.Rainfall_mm); % returns runoff_mm and volume

% Compare predicted peak volume to observed (if available)

if ismember('Observed_Runoff_m3s', events.Properties.VariableNames)

% Observed is m3/s - take durations into account for volume->peak comparisons (rough)

% We'll just show side-by-side for inspection:

disp(table(events.Date, events.Rainfall_mm, events.Observed_Runoff_m3s, ...

runoff.runoff_mm', runoff.runoff_volume_m3', 'VariableNames',...

{'Date','Rain_mm','Obs_Q_m3s','Pred_runoff_mm','Pred_runoff_vol_m3'}));

end

% Plot

scs.plotResults(events.Date, runoff);

## SOIL CONSERVATION SERVICE CURVE NUNMER METHOD (SCS)

It is a sub class implementing the SCS curve no. method for rainfall-runoff modeling

Initial abstraction (Ia): The model assumes initial losses equal 20% of the potential maximum retention(S)

Potential maximum retention (S): This value is determined by the curve number (CN) using the formulas

S = 1000CN - 10

Runoff (Q): The runoff volume is calculated using the formula

Q = (p-o.2s)^2p+0.8s  where (P) is the rainfall depth. This formula is valid only when P>0.2S).

classdef SCSModel < HydroModel

properties

CurveNumber % value 30..100 (user-supplied or estimated)

end

methods

function obj = SCSModel(catchment, CN)

obj@HydroModel(catchment);

obj.CurveNumber = CN;

end

function runoff = calculateRunoff(obj, rainfall_mm)

% SCS Curve Number method (simple event-based)

% S = (25400/CN - 254) in mm (S in mm if rainfall in mm)

CN = obj.CurveNumber;

S = (25400 ./ CN) - 254; % mm

Ia = 0.2 * S; % initial abstraction

P = rainfall_mm;

runoff_mm = zeros(size(P));

for i = 1:numel(P)

if P(i) > Ia

runoff_mm(i) = ((P(i) - Ia)^2) ./ (P(i) - Ia + S);

else

runoff_mm(i) = 0;

end

end

% convert runoff mm to m^3/s (approx) using catchment area and duration

area_m2 = obj.catchment.Area_m2;

% Assuming rainfall P is event total; to compute flow rate we'd need duration.

% We'll compute runoff volume (m^3) = runoff_mm/1000 * area_m2

runoff_volume_m3 = (runoff_mm ./ 1000) * area_m2;

% If you have Duration in hours (vector same length) you can compute m3/s:

runoff = struct('runoff_mm', runoff_mm, 'runoff_volume_m3', runoff_volume_m3);

end

function plotResults(obj, t, runoff)

figure;

subplot(2,1,1)

plot(t, runoff.runoff_mm); ylabel('Runoff (mm)'); xlabel('Event');

subplot(2,1,2)

plot(t, runoff.runoff_volume_m3); ylabel('Runoff volume (m^3)'); xlabel('Event');

sgtitle(sprintf('SCS model results (CN=%.1f)', obj.CurveNumber));

end

end

end

# 2.2.10 GRAPHICAL USER INTERFACE.

### This s an application we designed to run specific interface for the hydrological datasets of the project.   It was also to run different input data coded in to MATLAB.WE designed it in order to simplify and quickly have targeted data access about the project.

classdef hydroApp < handle

% A simple hydrology simulator GUI

properties

fig

panelLoad

panelResults

axDEM

axRain

axRunoff

btnLoadRain

btnLoadDEM

btnSimulate

rainData

demData

runoffData

end

methods

%% Constructor

function app = hydroApp()

close all;

app.buildUI();

end

%% Build UI Layout

function buildUI(app)

app.fig = figure('Name','Hydrology Simulator',...

'Position',[200 100 1100 600],...

'NumberTitle','off',...

'Color','white');

% Panels

app.panelLoad = uipanel(app.fig,'Title','Load Data',...

'Position',[0.01 0.55 0.25 0.44]);

app.panelResults = uipanel(app.fig,'Title','Results',...

'Position',[0.27 0.02 0.72 0.98]);

% Axes

app.axDEM = axes(app.panelResults,...

'Position',[0.05 0.55 0.4 0.4]);

title(app.axDEM,'Digital Elevation Model');

app.axRain = axes(app.panelResults,...

'Position',[0.55 0.55 0.4 0.4]);

title(app.axRain,'Rainfall Data');

app.axRunoff = axes(app.panelResults,...

'Position',[0.3 0.05 0.4 0.4]);

title(app.axRunoff,'Simulated Runoff');

% Buttons

app.btnLoadRain = uicontrol(app.panelLoad,'Style','pushbutton',...

'String','Load Rainfall Dataset',...

'FontSize',10,...

'Position',[30 180 200 40],...

'Callback',@(,)app.loadRainData());

app.btnLoadDEM = uicontrol(app.panelLoad,'Style','pushbutton',...

'String','Load DEM File',...

'FontSize',10,...

'Position',[30 120 200 40],...

'Callback',@(,)app.loadDEM());

app.btnSimulate = uicontrol(app.panelLoad,'Style','pushbutton',...

'String','Run Simulation',...

'FontSize',10,...

'Position',[30 40 200 50],...

'Enable','off',...

'Callback',@(,)app.simulateRunoff());

end

%% Load Rainfall Dataset

function loadRainData(app)

[file,path] = uigetfile({'.csv;.xlsx'},'Select Rainfall Data');

if isequal(file,0)

return;

end

fullFile = fullfile(path,file);

[ext] = fileparts(fullFile);

if strcmp(ext,'.csv')

app.rainData = readtable(fullFile);

else

app.rainData = readtable(fullFile);

end

% Plot rainfall data

plot(app.axRain, app.rainData{:,1}, app.rainData{:,2});

xlabel(app.axRain,'Time');

ylabel(app.axRain,'Rainfall (mm)');

title(app.axRain,'Rainfall Time Series');

app.enableSimulation();

end

%% Load DEM

function loadDEM(app)

[file,path] = uigetfile({'.tif;.asc'},'Select DEM File');

if isequal(file,0)

return;

end

fullFile = fullfile(path,file);

try

app.demData = readmatrix(fullFile);

catch

app.demData = load(fullFile);

end

imagesc(app.axDEM, app.demData);

colorbar(app.axDEM);

title(app.axDEM,'Digital Elevation Model');

app.enableSimulation();

end

%% Enable simulate button

function enableSimulation(app)

if ~isempty(app.rainData) && ~isempty(app.demData)

set(app.btnSimulate,'Enable','on');

end

end

%% Run Simple Runoff Simulation

function simulateRunoff(app)

rainfall = app.rainData{:,2};

% Simple SCS Curve Number Runoff model

CN = 75;

S = (25400/CN) - 254;

runoff = zeros(size(rainfall));

for i = 1:length(rainfall)

P = rainfall(i);

Ia = 0.2 * S;

if P > Ia

runoff(i) = (P - Ia)^2 / (P + 0.8*S);

else

runoff(i) = 0;

end

end

app.runoffData = runoff;

% Plot runoff

plot(app.axRunoff, app.rainData{:,1}, runoff, 'LineWidth', 1.5);

xlabel(app.axRunoff,'Time');

ylabel(app.axRunoff,'Runoff (mm)');

title(app.axRunoff,'Simulated Runoff');

end

end

end

# CONCLUSION

The assignment was successful since there was maximum cooperation among group 11 members. It exposed us to the different computation skills, engineering and design knowledge. Through this assignment we were exposed on different methods of data collection i.e. from Kaggle.com and how to manipulate and visualize it.

# REFERENCES

1. MATLAB PROGRAMMING by T. KIRANI SINGH & B.B. CHANDURI